Christie® MicroTiles™

AutoDesk® RevIt®

MicroTiles Types

June 3, 2011

MicroTile Types - Description:

- MicroTiles_Curved-Panel-Array: a curved array of microtiles (single row).

This array is meant to be mounted on the floor and it has feet available. To create a curved wall a manual vertical array will be necessary.

- MicroTiles_Floor-Mount-w-Hardware: single tile with feet to be mounted on the floor. Four feet options are available in this model: Right side end foot; right side center foot; left side end foot and left side center foot. A single tile can be used to create more complex arrays (irregular shapes with or without gaps). The array using this model has to be done manually.

- MicroTiles_Floor-Mount: single tile without feet. A single tile can be used to create more complex arrays (irregular shapes and gaps). The array using this model has to be done manually.

- MicroTiles_Linear-Panel-Array: for a single row or column to be mounted on the wall – anything more than one row or column use rectangular array. It has feet and brackets available

- MicroTiles_Microtiles-ECU-E100: single ECU unit.

- MicroTiles_Rectangular-Panel-Array: more than one column and row. To be mounted on the wall, it has feet and brackets available and the number of tiles can be selected for rows and columns. In this array brackets are turned on automatically when the panel is five or more rows high. Feet turn automatically on for the bottom row.

- MicroTiles_Wall-Mounted-w-Hardware:  single tile with bracket to be mounted on a wall. A single tile can be used to create more complex arrays (irregular shapes and gaps). The array using this model has to be done manually.

- MicroTiles_Wall-Mounted: single tile without bracket to be mounted on a wall. A single tile can be used to create more complex arrays (irregular shapes and gaps). The array using this model has to be done manually.

Please send us your comment about this page

How to...

... create a floor: create a new project, then insert floor from home tab. Select the shape (under create floor boundary), and draw it on the screen. Once the layout is done click on finish floor (this saves the changes and exits sketch mode).

... create a wall: in your existing project after the floor is drawn, insert wall from home tab. Draw the wall anywhere on the floor previously inserted. Press the esc button to end the wall command.

... insert a Christie model: Under families, electrical fixtures find the desired model drag and drop into your created space. Once this is done, select where the model will be mounted by clicking on a surface (wall or floor). A surface can be selected when its borders are highlighted in purple. This surface selection can be changed at any time by selecting the model and clicking on “pick new host” on the “modify electrical fixtures tab”.

... change level of detail: to reduce regeneration time when rendering, the level of details on a model can be reduced by selecting coarse on the bottom left corner of Revit (see screen capture below)

For more accurate rendering medium and fine detail levels are also available.

... change parameters: for all the models parameters can be found by selecting the model and then clicking on “element properties” on the left of the top menu under “modify electrical fixtures” tab. There are two types of parameters Instance and Type.

Instance properties will have options like turning feet on and off, turn brackets on and off, change radius on a curved array or the number of tiles, and change linear array type from horizontal to vertical. Another piece of information that can be found in this screen is the energy consumption for arrays.

Type properties contain the metadata, including power, weight, link to spec sheet, brightness, dimensions, etc. Some of these parameters can be changed for the current project if desired.

... change model type: model type can be changed by selecting the model already on the screen and then selecting the new model from the drop down list “change element type” under “modify electrical fixtures” tab. For example after inserting a MicroTiles_Wall-Mounted-w-Hardware (single tile with bracket) we realize we just need a column array. By selecting the single tile and changing the element type to MicroTiles_Linear-Panel-Array the change becomes easier.

... create a custom shape array: insert a single tile, for example a MicroTiles_Wall-Mounted-w-Hardware (single tile with bracket)  on a wall following the steps provided before.

Before customizing, create a basic rectangular array manually. To create an array, click on the tile and then on “array” under “modify electrical fixtures”. Specify the number of tiles to be added to the array (see picture below), and then select the spacing and position by clicking on the screen. Arrays can be done in the horizontal direction as well as in the vertical direction.

Tip: the current tile can be used as guide to place the array (see next page).

Array Direction

Start

End

If you wish to delete a tile out of the middle to make a skyscraper or hollow centre array; get rid of a bracket or even add feet, the first step is to ungroup the manually arrayed tiles.

To ungroup a tile, select the tile and then click on “ungroup” under “modify model groups tab”. Note: This tab will only appear after selecting the tile if and only if the tile is in a group or array. Once ungrouped you can turn on or off feet or brackets on that specific tile or delete it (see details below).

Ungroup breaks the link between a tile and its array.

The tile that has been ungrouped can now be modified or deleted.

If desired hardware can be turned on or off after ungrouping a tile.